Rev. 4/01

| COUNTY ST. LUCIE |  | PROJECT Hydrology – UEC Floridian Wells |  |  | DESIGNATION STL278 2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # SECTION 12 |  | # TOWNSHIP 36 SOUTH |  |  | # RANGE 40 EAST |  |  |
| GEOGRAPHIC INDEX OF QUAD |  |  |  |  |  |  |  |
| Established by Nick Miller Inc. Recovered by |  |  | NAME OF QUADRANGLE ANKONA |  |  |  |  |
| SURVEYOR Stephen M. Gordon DATE 1/18/2005 |  |  | FIELD BOOK 1 PAGE 15 |  |  |  |  |
| HORIZONTAL DATUM: 1927 1983 Other__________ (circle one) ZONE E or W |  |  |  |  |  |  |  |
| VERTICAL DATUM: MSL 1929 1988 Other _________ (circle one) |  |  |  |  |  |  |  |
| CONTROL ACCURACY: HORIZONTAL 1 2 3 SUB-METER (circle one) VERTICAL 1 2 3 |  |  |  |  |  |  |  |
| # STATE PLANE COORDINATES | # E 884655ft |  |  | # N 1098161ft |  | EL. 15.46 ft |  |
| LATITUDE N 27.35306° LONGITUDE W 80.29609° |  |  |  |  |  |  |  |
| DESCRIPTION |  |  |  |  |  |  |  |
| To Reach: TO REACH MARK FROM THE JUNCTION OF US HIGHWAY 1 AND STATE ROAD 716 (PORT SAINT LUCIE BLVD) IN PORT SAINT LUCIE, GO NORTH ON US HIGHWAY 1 FOR 6.0 MI (9.66 KM) TO EASY STREET, TURN RIGHT ON EASY STREET AND GO EAST FOR 1.7 MI (2.74 KM) TO DIRT ROAD, GO EAST ON DIRT ROAD FOR 100 FT (30.48 M) TO SFWMD LOCKED GATE, CONTINUE TO GO EAST THROUGH GATE ON DIRT ROAD FOR 12 FT (3.66 M) TO MARK ON THE RIGHT, SET IN TOP OF A ROUND CONCRETE MONUMENT 0.2 FT (6.1 CM) BELOW LEVEL OF THE GROUND. LOCATED 8.6 FT (2.62 M) WEST OF STL 278 MONITORING WELL, 13.3 FT (4.05 M) SOUTHEAST OF SOUTHERN GATE POST, 24 FT (7.32 M) SOUTHWEST OF WESTERN MOST POST OF CONTROL TOWER WITH SOLAR PANEL, 4 FT (1.22 M) EAST OF BARBED WIRE FENCE. |  |  |  |  |  |  |  |
| Notable Land marks: |  |  |  |  |  |  |  |

| SKETCH |  |
| --- | --- |