Rev. 4/01

| COUNTY MIAMI-DADE |  | PROJECT SOUTH DISTRICT WASTEWATER TREATMENT PLANT |  |  | DESIGNATION MDWASD-S |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # SECTION 21 |  | # TOWNSHIP 56S |  |  | # RANGE 40E |  |  |
| GEOGRAPHIC INDEX OF QUAD Florida |  |  |  |  |  |  |  |
| Established by Biscayne Engineering Company, Inc. |  |  | NAME OF QUADRANGLE 1303 |  |  |  |  |
| SURVEYOR Mike J. Bartholomew DATE 10 / 26 / 2004 |  |  | FIELD BOOK 2525 PAGE 08-22 2516 PAGE 27-30 |  |  |  |  |
| HORIZONTAL DATUM: 1927 1983 Other__________ (circle one) ZONE 0901 (EAST) |  |  |  |  |  |  |  |
| VERTICAL DATUM: MSL 1929 1988 Other _________ (circle one) |  |  |  |  |  |  |  |
| CONTROL ACCURACY: HORIZONTAL 1 2 3 SUB-METER (circle one) VERTICAL 1 2 3 |  |  |  |  |  |  |  |
| # STATE PLANE COORDINATES MDWASD-S (U.S. Survey feet) | # X= 871481.893 |  |  | # Y= 443171.176 |  | EL.= 8.81’ (NAVD-88) EL= 10.33’ (NGVD-29) |  |
| LATITUDE MDWASD-S 25º33’04.97”N LONGITUDE 080º20’48.91”W |  |  |  |  |  |  |  |
| DESCRIPTION |  |  |  |  |  |  |  |
| To Reach: From the intersection of US1 and SW 248th Street go EAST along the SW 248th Street to the EAST for 3.95 |  |  |  |  |  |  |  |
| miles to intersection with SW 97th Avenue; THENCE go NORTH along the SW 97th Avenue for 1 mile to intersection with |  |  |  |  |  |  |  |
| SW 232nd Street; THENCE go EAST along the SW 232nd Street for 0.9 mile to the intersection with SW 88th Avenue; |  |  |  |  |  |  |  |
| THENCE go SOUTH along the SW 88th Avenue for 0.05 mile to a guardhouse at the entrance to the South District Wastewater Treatment Plant. Meander Southerly and Westerly along a service road near the perimeter of the facility |  |  |  |  |  |  |  |
| for 0.8 mile (+/-) to the station MDWASD-S near the Northwest corner of the facility. |  |  |  |  |  |  |  |
| MDWASD-S is standard South Florida Water Management District Aluminum Disk (stamped “MDWASD-S”), set in the Northeast corner of a concrete slab for wellheads. Set 19.65 feet West of and 36.00 feet South of the Northeast corner of a concrete platform for wellheads. |  |  |  |  |  |  |  |
| Note: Origin of NAVD-88 elevation for BM "MDWASD-S" is closed bench level circuit through NGS benchmarks AH2270 and AH2272 using published NAVD-88 values. Origin of NGVD-29 elevation for BM "MDWASD-S" is conversion of NAVD-88 value using NGS Vertcon (on NGS web site) |  |  |  |  |  |  |  |

SKETCH: SEE PAGE 2