Rev. 4/01

| COUNTY PALM BEACH |  | PROJECT Hydrology – UEC Floridian Wells |  |  | DESIGNATION PB1648 2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # SECTION 6 |  | # TOWNSHIP 41 SOUTH |  |  | # RANGE 42 EAST |  |  |
| GEOGRAPHIC INDEX OF QUAD |  |  |  |  |  |  |  |
| Established by Nick Miller, Inc. Recovered by |  |  | NAME OF QUADRANGLE ROOD |  |  |  |  |
| SURVEYOR Stephen M. Gordon DATE 12/20/2004 |  |  | FIELD BOOK 1 PAGE 5 |  |  |  |  |
| HORIZONTAL DATUM: 1927 1983 Other__________ (circle one) ZONE E or W |  |  |  |  |  |  |  |
| VERTICAL DATUM: MSL 1929 1988 Other _________ (circle one) |  |  |  |  |  |  |  |
| CONTROL ACCURACY: HORIZONTAL 1 2 3 sub-meter (circle one) VERTICAL 1 2 3 |  |  |  |  |  |  |  |
| # STATE PLANE COORDINATES | # E 924624 ft |  |  | # N 946812 ft |  | EL. 13.96 ft |  |
| LATITUDE N 26.93607° LONGITUDE W 80.17604° |  |  |  |  |  |  |  |
| DESCRIPTION |  |  |  |  |  |  |  |
| To Reach: TO REACH MARK FROM THE JUNCTION OF INTERSTATE 95 AND STATE ROAD 706 (INDIANTOWN ROAD) IN JUPITER, GO WEST ON STATE ROAD 706 (INDIANTOWN ROAD) FOR 1.6 MI (2.58 KM) TO ENTRANCE OF RIVERBEND PARK, TURN LEFT ON PAVED ROAD AND GO SOUTH FOR 0.2 MI (0.32 KM) TO LOCKED GATE ON RIGHT (BEFORE RIVERBEND PARK OFFICE TRAILERS), TURN RIGHT ON DIRT ROAD AND GO WESTERLY AND NORTHERLY FOR 0.2 MI (0.32 KM) TO (NORTHWEST CORNER OF PROPERTY) MARK ON LEFT, SET IN TOP OF A ROUND CONCRETE MONUMENT 0.2 FT (6.1 CM) BELOW LEVEL OF THE GROUND. LOCATED 91.5 FT(27.89M) SOUTHEAST FIRE HYDRANT, 8.0 FT (2.44 M) NORTHEAST OF WEST MONITOING WELL (PB1648), 8.2 FT (2.50 M) NORTH OF EAST MONITORING WELL (PB1649). |  |  |  |  |  |  |  |
| Notable Land marks: |  |  |  |  |  |  |  |

| SKETCH |  |
| --- | --- |