LITHOLOGIC WELL LOG PRINTOUT                   SOURCE - FGS

WELL NUMBER:   W-17013                         COUNTY -   BROWARD

TOTAL DEPTH:    2218  FT.                      LOCATION:  T.49S R.42E S.20

332 SAMPLES FROM 0 TO 2218 FT.                            LAT = 26D 10M 23S

LON = 80D 10M 44S

COMPLETION DATE: 02/00/93                      ELEVATION: 20 FT

OTHER TYPES OF LOGS AVAILABLE - OTHER

OWNER/DRILLER:SOUTH FLORIDA WATER MANAGEMENT DISTRICT/HENRY BLAIR

WORKED BY:__LI LI (10/15/93)

SFWMD ID# FOR CUTTINGS IS 011-18 (HOLE #HWTW-1), BROWARD COUNTY.

SFWMD GEOPHYSICAL LOG # 011000009.

LOCATED IN THE SW 1/4, SW 1/4, NE 1/4, SEC.20, T49S, R42E.

FLORIDA POLYCONIC EAST ZONE IN FEET; PLANAR X=769323; PLANAR Y=669411.

SFWMD GEOLOGIST LOGS FOR THIS MONITOR WELL.

WELL IS LOCATED IN THE FORT LAUDERDALE NORTH 7.5 MINUTE QUADRANGLE.

THE OKEECHOBEE FORMATION IS PROPOSED FOR THE PLIO-PLEISTOCENE INTERVAL

(SCOTT, 1992, P. 23, FLORIDA GEOLOGICAL SURVEY SPECIAL PUBLICATION 36).

0.   -   30.   000NOSM   NO SAMPLES

30.   -   70.   112MIMI   MIAMI LIMESTONE

70.   -  361.   121PCPC   PLIOCENE-PLEISTOCENE

361.   - 1035.   122HTRN   HAWTHORN GROUP

1035.   - 2218.   124AVPK   AVON PARK FM.

0.   -   30.   000NOSM   NO SAMPLES

55.   -   60.   000NOSM   NO SAMPLES

90.   -  100.   000NOSM   NO SAMPLES

160.   -  165.   000NOSM   NO SAMPLES

225.   -  230.   000NOSM   NO SAMPLES

315.   -  325.   000NOSM   NO SAMPLES

394.   -  395.   000NOSM   NO SAMPLES

515.   -  520.   000NOSM   NO SAMPLES

860.   -  870.   000NOSM   NO SAMPLES

1000.   - 1015.   000NOSM   NO SAMPLES

1590.   - 1593.   000NOSM   NO SAMPLES

1595.   - 1597.   000NOSM   NO SAMPLES

0  -   30   NO SAMPLES

30  -   55   LIMESTONE; VERY LIGHT ORANGE TO VERY LIGHT GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: OOLITE, CRYSTALS, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: MICROCRYSTALLINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-10%, SHELL-05%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

55  -   60   NO SAMPLES

60  -   70   LIMESTONE; VERY LIGHT ORANGE TO VERY LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: OOLITE, CRYSTALS, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-20%, SHELL-10%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

70  -   90   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-30%, SHELL-05%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

90  -  100   NO SAMPLES

100  -  115   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-20%, SHELL-20%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

115  -  120   LIMESTONE; YELLOWISH GRAY TO LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-30%, SHELL-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

120  -  140   LIMESTONE; YELLOWISH GRAY TO LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-20%, SHELL-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

140  -  160   LIMESTONE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-20%, SHELL-15%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

160  -  165   NO SAMPLES

165  -  175   LIMESTONE; VERY LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-20%, SHELL-15%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

175  -  178   LIMESTONE; LIGHT GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO COARSE

GOOD INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-20%, QUARTZ SAND-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

178  -  185   LIMESTONE; VERY LIGHT ORANGE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-15%, QUARTZ SAND-05%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

185  -  195   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-30%, QUARTZ SAND-20%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

195  -  200   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-10%, SHELL-10%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

200  -  208   LIMESTONE; YELLOWISH GRAY TO PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-10%, SHELL-05%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

208  -  225   LIMESTONE; YELLOWISH GRAY TO PINKISH GRAY

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-30%, QUARTZ SAND-05%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

225  -  230   NO SAMPLES

230  -  240   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-25%, QUARTZ SAND-05%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

240  -  260   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-10%, QUARTZ SAND-05%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

260  -  268   LIMESTONE; LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, QUARTZ SAND-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

268  -  285   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: SHELL-20%, QUARTZ SAND-20%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

AMMONIA SP.

285  -  298   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-30%, QUARTZ SAND-15%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, SPICULES, BENTHIC FORAMINIFERA

298  -  315   LIMESTONE; YELLOWISH GRAY

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-30%, QUARTZ SAND-20%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

315  -  325   NO SAMPLES

325  -  332   LIMESTONE; YELLOWISH GRAY

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-40%, QUARTZ SAND-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

332  -  361   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO VERY COARSE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-10%, SHELL-05%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

361  -  394   SANDSTONE; LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN SIZE: FINE; RANGE: VERY FINE TO MEDIUM

MEDIUM SPHERICITY; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: CALCILUTITE-25%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

ARCHAIAS SP., GYPSINA GLOBULA

394  -  395   NO SAMPLES

395  -  400   SHELL BED; WHITE TO YELLOWISH GRAY

35% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

UNCONSOLIDATED

ACCESSORY MINERALS: CALCITE-30%, QUARTZ SAND-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

OOLITE FRAGMENTS 10%; PROBABLY FROM CAVING.

400  -  410   LIMESTONE; VERY LIGHT ORANGE TO YELLOWISH GRAY

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

POOR INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-40%, QUARTZ SAND-10%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

410  -  415   LIMESTONE; YELLOWISH GRAY

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

POOR INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-25%, QUARTZ SAND-20%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

415  -  423   LIMESTONE; YELLOWISH GRAY TO LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-25%, SHELL-10%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

423  -  445   SILT; LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: CALCILUTITE-20%, QUARTZ SAND-05%

SHELL-05%, PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

ARCHAIAS SP.;  OOLITE FRAGMENTS MAY COME FROM CAVING.

445  -  450   SILT; LIGHT OLIVE GRAY TO YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: CALCILUTITE-20%, QUARTZ SAND-05%

SHELL-05%, PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

OOLITE FRAGMENTS MAY COME FROM CAVING.

450  -  456   SILT; LIGHT OLIVE GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, CLAY MATRIX

ACCESSORY MINERALS: CALCILUTITE-25%, QUARTZ SAND-03%

SHELL-02%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

OOLITE FRAGMENTS MAY COME FROM CAVING.

456  -  470   LIMESTONE; YELLOWISH GRAY TO LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-30%, SHELL-15%

PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

ARCHAIAS SP.

470  -  480   LIMESTONE; YELLOWISH GRAY TO LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-25%, SHELL-10%

PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS

480  -  515   LIMESTONE; YELLOWISH GRAY TO LIGHT OLIVE GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: FINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-40%, SHELL-05%

PHOSPHATIC SAND-03%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

515  -  520   NO SAMPLES

520  -  540   SILT; LIGHT OLIVE GRAY

20% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, CLAY MATRIX

ACCESSORY MINERALS: CALCILUTITE-25%, PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

540  -  550   LIMESTONE; YELLOWISH GRAY TO LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CRYSTALS, BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

ACCESSORY MINERALS: QUARTZ SAND-25%, SHELL-20%

PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, BENTHIC FORAMINIFERA

550  -  570   LIMESTONE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-20%, QUARTZ SAND-10%

PHOSPHATIC SAND-03%

FOSSILS: MOLLUSKS

570  -  600   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, CRYSTALS

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

600  -  640   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, CRYSTALS

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO MEDIUM; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-01%

FOSSILS: FOSSIL FRAGMENTS

640  -  700   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, CRYSTALS

25% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-02%, PHOSPHATIC SAND-02%

FOSSILS: FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

700  -  725   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, SPICULES

725  -  755   CALCILUTITE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO VERY COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-25%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

755  -  770   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, PHOSPHATIC SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

770  -  795   CALCILUTITE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-20%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

795  -  820   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 25% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-05%

DOLOMITE-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

820  -  824   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-10%, SHELL-05%

PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

824  -  860   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-03%

DOLOMITE-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

860  -  870   NO SAMPLES

870  -  890   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-03%

DOLOMITE-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS

890  -  920   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-03%, PHOSPHATIC SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

920  -  930   CALCILUTITE; YELLOWISH GRAY

10% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: PHOSPHATIC SAND-02%

FOSSILS: FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

930  -  945   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, QUARTZ SAND-05%

PHOSPHATIC SAND-02%, DOLOMITE-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

945  -  950   LIMESTONE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, PHOSPHATIC SAND-08%

QUARTZ SAND-05%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

950  -  965   CALCILUTITE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-01%

QUARTZ SAND-01%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

965  -  980   LIMESTONE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: PHOSPHATIC SAND-10%, SHELL-05%

QUARTZ SAND-02%

FOSSILS: MOLLUSKS, FOSSIL FRAGMENTS, BENTHIC FORAMINIFERA

980  -  990   CALCILUTITE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-05%, PHOSPHATIC SAND-03%

QUARTZ SAND-02%

FOSSILS: MOLLUSKS, SHARKS TEETH, BENTHIC FORAMINIFERA

990  - 1000   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-20%, PHOSPHATIC SAND-02%

QUARTZ SAND-01%

FOSSILS: MOLLUSKS, SPICULES, BENTHIC FORAMINIFERA

1000  - 1015   NO SAMPLES

1015  - 1025   LIMESTONE; VERY LIGHT GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO GRANULE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%

FOSSILS: MOLLUSKS

1025  - 1030   LIMESTONE; VERY LIGHT GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC, CRYSTALS

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO GRANULE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: SHELL-10%, QUARTZ SAND-10%

FOSSILS: MOLLUSKS

1030  - 1035   LIMESTONE; VERY LIGHT GRAY TO LIGHT OLIVE GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: QUARTZ SAND-15%, SHELL-10%

PHOSPHATIC SAND-05%

FOSSILS: MOLLUSKS

BOTTOM OF THE HAWTHORN FORMATION.

1035  - 1060   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MOLLUSKS, ECHINOID, MILIOLIDS

BENTHIC FORAMINIFERA

DICTYOCONUS COOKEI, DICTYOCONUS AMERICANUS, GYPSINA

GLOBULA, CRIBROBULIMINA SP., LITUONELLA SP.

1060  - 1090   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS, CALCILUTITE

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT, CALCILUTITE MATRIX

FOSSILS: CORAL, ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1090  - 1125   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO VERY COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: SPICULES, ECHINOID, MILIOLIDS

BENTHIC FORAMINIFERA

POOR SAMPLES FROM 1110' TO 1125'; GRAY CALCILUTITE APPEARS

DUE TO CAVING.

1125  - 1130   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: SPICULES, MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS COOKEI, DICTYOCONUS AMERICANUS, SPIROLINA SP.

CRIBROBULIMINA SP.

1130  - 1160   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1160  - 1175   LIMESTONE; PINKISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1175  - 1200   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

50% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1200  - 1221   CALCILUTITE; VERY LIGHT ORANGE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1221  - 1250   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-05%

FOSSILS: MOLLUSKS, ECHINOID, MILIOLIDS

BENTHIC FORAMINIFERA

1250  - 1280   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1280  - 1290   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: DOLOMITE-05%

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1290  - 1305   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO VERY COARSE

MODERATE INDURATION

CEMENT TYPE(S): SPARRY CALCITE CEMENT

ACCESSORY MINERALS: DOLOMITE-05%

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

1305  - 1335   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-02%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS AMERICANUS, DICTYOCONUS COOKEI, SPIROLINA SP.

CRIBROBULIMINA SP., LITUONELLA SP.

1335  - 1355   LIMESTONE; WHITE TO PINKISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

50% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MOLLUSKS, MILIOLIDS, BENTHIC FORAMINIFERA

1355  - 1375   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1375  - 1380   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-02%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1380  - 1385   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-02%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1385  - 1405   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-02%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1405  - 1420   LIMESTONE; PINKISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1420  - 1430   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CRYSTALS

90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, SPARRY CALCITE CEMENT

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1430  - 1440   CALCILUTITE; PINKISH GRAY

10% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO VERY FINE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1440  - 1470   LIMESTONE; PINKISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1470  - 1490   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1490  - 1520   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

50% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-03%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1520  - 1530   LIMESTONE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1530  - 1534   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1534  - 1565   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1565  - 1570   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1570  - 1580   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1580  - 1590   LIMESTONE; WHITE

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO VERY FINE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1590  - 1593   NO SAMPLES

1593  - 1595   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: ECHINOID, MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS AMERICANUS, DICTYOCONUS COOKEI, CRIBROBULIMINA

SP.

1595  - 1597   NO SAMPLES

1597  - 1605   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1605  - 1610   LIMESTONE; WHITE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1610  - 1615   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO VERY COARSE

UNCONSOLIDATED

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS AMERICANUS, DICTYOCONUS COOKEI, CRIBROBULIMINA

SP., LITUONELLA SP., SPIROLINA SP.

1615  - 1620   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

UNCONSOLIDATED

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1620  - 1665   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1665  - 1700   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1700  - 1703   LIMESTONE; VERY LIGHT GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 20% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO MEDIUM; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1703  - 1708   LIMESTONE; YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1708  - 1730   LIMESTONE; PINKISH GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO MEDIUM; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1730  - 1735   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1735  - 1740   LIMESTONE; WHITE TO PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1740  - 1745   LIMESTONE; WHITE TO VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1745  - 1750   LIMESTONE; WHITE

25% POROSITY: INTERGRANULAR, PIN POINT VUGS

POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1750  - 1760   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1760  - 1770   LIMESTONE; VERY LIGHT ORANGE

15% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO COARSE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1770  - 1800   LIMESTONE; VERY LIGHT ORANGE

30% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL; POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1800  - 1805   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS AMERICANUS, DICTYOCONUS COOKEI, COSKINOLINA

SP.

1805  - 1810   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1810  - 1820   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC; 90% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1820  - 1830   LIMESTONE; WHITE TO VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1830  - 1835   LIMESTONE; PINKISH GRAY

20% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC, CRYSTALS

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE

RANGE: MICROCRYSTALLINE TO VERY COARSE; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1835  - 1845   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1845  - 1855   LIMESTONE; WHITE TO YELLOWISH GRAY

20% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: MICROCRYSTALLINE TO VERY COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1855  - 1870   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1870  - 1880   LIMESTONE; YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO COARSE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1880  - 1885   LIMESTONE; VERY LIGHT ORANGE

20% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1885  - 1890   LIMESTONE; WHITE TO YELLOWISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

75% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1890  - 1900   LIMESTONE; WHITE TO VERY LIGHT GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1900  - 1910   LIMESTONE; WHITE TO VERY LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1910  - 1920   CALCILUTITE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

25% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1920  - 1938   LIMESTONE; WHITE TO PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRAVEL

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1938  - 1940   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1940  - 1945   LIMESTONE; VERY LIGHT GRAY

20% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO GRANULE; MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1945  - 1970   LIMESTONE; WHITE TO PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1970  - 1975   LIMESTONE; WHITE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1975  - 1977   LIMESTONE; PINKISH GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: COARSE; RANGE: VERY FINE TO GRAVEL

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-03%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

1977  - 1980   DOLOSTONE; LIGHT GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

FOSSILS: BENTHIC FORAMINIFERA

1980  - 1999   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE; 40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: BENTHIC FORAMINIFERA

1999  - 2010   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2010  - 2025   LIMESTONE; WHITE TO PINKISH GRAY

25% POROSITY: INTERGRANULAR, PIN POINT VUGS

POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MEDIUM; RANGE: VERY FINE TO GRAVEL

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-02%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2025  - 2028   DOLOSTONE; WHITE TO LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

50-90% ALTERED; EUHEDRAL

GRAIN SIZE: FINE; RANGE: VERY FINE TO MEDIUM

GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX, DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-40%

2028  - 2040   DOLOSTONE; VERY LIGHT ORANGE TO GRAYISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-20%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2040  - 2045   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

85% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

POOR INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-05%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

DICTYOCONUS AMERICANUS, DICTYOCONUS COOKEI, COSKINOLINA

SP., CRIBROBULIMINA SP.

2045  - 2050   DOLOSTONE; LIGHT GRAY TO VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

10-50% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-40%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2050  - 2060   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-30%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2060  - 2080   LIMESTONE; VERY LIGHT ORANGE

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

70% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-03%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2080  - 2082   DOLOSTONE; VERY LIGHT ORANGE TO GRAYISH BROWN

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

50-90% ALTERED; SUBHEDRAL

GRAIN SIZE: FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT, CALCILUTITE MATRIX

ACCESSORY MINERALS: CALCITE-40%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2082  - 2085   LIMESTONE; YELLOWISH GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC, CRYSTALS

30% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO MEDIUM; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-10%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2085  - 2087   DOLOSTONE; LIGHT GRAY TO MODERATE GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-40%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2087  - 2089   DOLOSTONE; GRAYISH BROWN

15% POROSITY: INTERGRANULAR, PIN POINT VUGS

LOW PERMEABILITY; 50-90% ALTERED; EUHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

FOSSILS: BENTHIC FORAMINIFERA

2089  - 2091   LIMESTONE; WHITE TO GRAYISH ORANGE PINK

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

40% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-20%

FOSSILS: BENTHIC FORAMINIFERA

2091  - 2095   DOLOSTONE; WHITE TO GRAYISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

10-50% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-50%

FOSSILS: BENTHIC FORAMINIFERA

2095  - 2100   DOLOSTONE; GRAYISH BROWN TO GRAYISH ORANGE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

90-100% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2100  - 2105   LIMESTONE; LIGHT GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: CALCILUTITE, BIOGENIC

25% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO MEDIUM; GOOD INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-10%

FOSSILS: BENTHIC FORAMINIFERA

2105  - 2115   DOLOSTONE; LIGHT GRAYISH BROWN

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

90-100% ALTERED; ANHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-05%

2115  - 2120   DOLOSTONE; LIGHT GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-15%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2120  - 2130   DOLOSTONE; MODERATE YELLOWISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; EUHEDRAL

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

2130  - 2137   DOLOSTONE; DARK YELLOWISH BROWN TO MODERATE YELLOWISH BROWN

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

90-100% ALTERED; EUHEDRAL

GRAIN SIZE: FINE; RANGE: VERY FINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

2137  - 2148   DOLOSTONE; GRAYISH BROWN

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

90-100% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO VERY FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-02%

2148  - 2150   DOLOSTONE; LIGHT GRAY TO DARK GRAY

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; EUHEDRAL

GRAIN SIZE: FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-20%

2150  - 2154   DOLOSTONE; MODERATE YELLOWISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

90-100% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: VERY FINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

2154  - 2159   DOLOSTONE; LIGHT GRAY TO DARK GRAY

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

FOSSILS: BENTHIC FORAMINIFERA

2159  - 2160   DOLOSTONE; DARK YELLOWISH BROWN TO DARK YELLOWISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

90-100% ALTERED; SUBHEDRAL

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO FINE

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

2160  - 2166   DOLOSTONE; GRAYISH BROWN

15% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; ANHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: CRYPTOCRYSTALLINE TO VERY FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-10%

2166  - 2170   DOLOSTONE; GRAYISH BROWN

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

90-100% ALTERED; EUHEDRAL

GRAIN SIZE: FINE; RANGE: VERY FINE TO MEDIUM

GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

2170  - 2180   DOLOSTONE; LIGHT GRAY TO GRAYISH BROWN

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

50-90% ALTERED; SUBHEDRAL

GRAIN SIZE: MICROCRYSTALLINE

RANGE: MICROCRYSTALLINE TO FINE; GOOD INDURATION

CEMENT TYPE(S): DOLOMITE CEMENT

ACCESSORY MINERALS: CALCITE-25%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2180  - 2195   LIMESTONE; VERY LIGHT ORANGE TO LIGHT GRAY

25% POROSITY: INTERGRANULAR, POSSIBLY HIGH PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

80% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: FINE; RANGE: VERY FINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-25%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2195  - 2218   LIMESTONE; WHITE

20% POROSITY: INTERGRANULAR, LOW PERMEABILITY

GRAIN TYPE: BIOGENIC, CALCILUTITE

60% ALLOCHEMICAL CONSTITUENTS

GRAIN SIZE: VERY FINE; RANGE: MICROCRYSTALLINE TO GRANULE

MODERATE INDURATION

CEMENT TYPE(S): CALCILUTITE MATRIX

ACCESSORY MINERALS: DOLOMITE-05%

FOSSILS: MILIOLIDS, BENTHIC FORAMINIFERA

2218  TOTAL DEPTH