| APPENDIX A |  |  |
| --- | --- | --- |
| Lithologic Log – R.D. Keene Park Orange County, Florida |  |  |
| Depth in Feet (bls) |  | ORF -61 R.D. Keene Park |
| Lithologic Log |  |  |
| From | To | Lithologic Description |
| 0 | 10 | Grey/tan, well sorted medium quartz sand, good permeability |
| 10 | 20 | Dark brown,medium to coarse sand, organic material |
| 20 | 30 | Light brown,fine quartz sand, trace of clay |
| 30 | 40 | Light brown,fine grain silty sand, trace of clay |
| 40 | 50 | Grey, fine grain sand with trace of clay |
| 50 | 60 | Grey,fine sand with clay |
| 60 | 85 | Grey siltstone, well indurated, some poorly sorted quartz sand, broken shark tooth |
| 85 | 90 | Grey sandy clay with well indurated quartz sand |
| 90 | 100 | Grey siltstone/mudstone with quartz sand |
| 100 | 106 | Grey mudstone |
| 106 | 120 | Void, no samples |
| 120 | 130 | Light tan, packstone with broken shell |
| 130 | 150 | Light tan, pachstone with shell fragements |
| 150 | 155 | Light tan wackstone with shell and grey siltstone |
| 155 | 165 | Light grey lime mudstone, |
| 165 | 170 | Light grey to tan wackstone |
| 170 | 180 | Light grey mudstone with fossils |
| 180 | 200 | Light grey to tan wackestone with increasing fossil fragments |
| 200 | 215 | Light brown packestone, moldic porosity |
| 215 | 235 | Tan mudstone with broken echinoid fossils |
| 235 | 240 | Brown to dark brown dolostone with voids, moldic porosity |
| 240 | 250 | Light brown packestone with voids |
| 250 | 260 | Brown packestone with voids |
| 260 | 275 | Light brown packestone skeletal matter visible, fossils not identified, effervesses with 10% HCl. |
| 275 | 285 | Light brown grainstone, echinoid fragments, moldic porosity |
| 285 | 310 | Light brown packestone, echinoid fragements, friable |
| 310 | 315 | Light brown packestone with cone shape(possible dictyoconus) fossils |
| 315 | 320 | Light brown grainstone, fossil fragements |
| 320 | 335 | Light brown mudstone with fossil fragements |
| 335 | 340 | Tan grainstone with fossil frags(dictyoconus) |
| 340 | 350 | Light brown grainstone, fossil fragements with brown dolomite present |
| 350 | 360 | Light brown grainstone, fossil fragments(dictyoconus) less dolomite |
| 360 | 365 | Brown dolomite, hard |
| 365 | 375 | Tan lime mudstone, |
| 375 | 390 | Cream-colored grainstone, friable, moldic and intergranular porosity |
| 390 | 500 | Samples missing |
| 500 | 525 | Tan wackestone with small voids present |
| 525 | 535 | Tan mudstone |
| 535 | 540 | Tan mudstone |
| 540 | 545 | Tan packstone, fossil fragements, efferveses with 10%HCl |
| 545 | 555 | Light brown dolostone/mudstone |
| 555 | 560 | Brown wackestone |
| 560 | 565 | Light brown dolostone /wackestone with small solution voids |
| 565 | 580 | Tan mudstone, vuggy and moldic porosity |
| 580 | 605 | Brown dolostone/wackestone with solution voids |
| 605 | 620 | Dark brown dolostone/wackestone with solution voids |
| 620 | 625 | Light brown mudstone with grey well indurated siltstone |
| 625 | 650 | Brown dolostone/wackestone with solution voids visible |

APPENDIX B - Geophysical logs from ORF-61

GAM(NAT)

API-GR

0

150

SP

MV

-160

160

CALIPER

INCH

10

20

DEPTH

FT

LATERAL

OHM-M

2

2000

RES(64N)

OHM-M

2

2000

RES(16N)

OHM-M

2

2000

BHC-DELT

USEC/FT

140

40

S_TEMP

DEG

70

80

S_RES(F)

OHM-M

0

40

TEMP

DEG

70

80

S-FLOW

RPM

0

300

FLOW

RPM

0

300

100

200

300

400

500

600