Rev. 4/01

| COUNTY OSCEOLA |  | PROJECT Hydrology – UEC Floridian Wells |  |  | DESIGNATION YEEHAW 2005 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| # SECTION 14 |  | # TOWNSHIP 32 SOUTH |  |  | # RANGE 34 EAST |  |  |
| GEOGRAPHIC INDEX OF QUAD |  |  |  |  |  |  |  |
| Established by Nick Miller Inc. Recovered by |  |  | NAME OF QUADRANGLE FT DRUM NORTHWEST |  |  |  |  |
| SURVEYOR Stephen M. Gordon DATE 1/12/2005 |  |  | FIELD BOOK 1 PAGE 28 |  |  |  |  |
| HORIZONTAL DATUM: 1927 1983 Other__________ (circle one) ZONE E or W |  |  |  |  |  |  |  |
| VERTICAL DATUM: MSL 1929 1988 Other _________ (circle one) |  |  |  |  |  |  |  |
| CONTROL ACCURACY: HORIZONTAL 1 2 3 SUB-METER (circle one) VERTICAL 1 2 3 |  |  |  |  |  |  |  |
| # STATE PLANE COORDINATES | # E 689775ft |  |  | # N 1222460ft |  | EL. 58.59 ft |  |
| LATITUDE N 27.69672° LONGITUDE W 80.89614° |  |  |  |  |  |  |  |
| DESCRIPTION |  |  |  |  |  |  |  |
| To Reach: TO REACH MARK FROM US HIGHWAY 441 AND STATE ROAD 60, GO EAST ON STATE ROAD 60 FOR 0.4 MI (30.48 KM) TO TURNPIKE ENTRANCE ROAD, TURN LEFT ON TURNPIKE ENTRANCE ROAD AND GO NORTH FOR 100 FT (30.48 M) TO TURNPIKE UTILITY ROAD, TURN RIGHT ON UTILITY ROAD AND GO EAST FOR 0.2 MI (0.32 KM) TO MARK ON LEFT, SET IN TOP OF A ROUND CONCRETE MONUMENT 0.2 FT (6.1 CM) BELOW LEVEL OF THE GROUND. LOCATED 7.35 FT (2.24 M) EAST OF NORTHEAST MONITORING WELL, 124.7 FT (38.0 M) WEST OF CONTROL BOX WITH SOLAR PANEL AND RADIO TOWER, 126.6 FT (38.59 M) NORTHEAST OF UTILITY POLE (17/426/8), 181.2 FT (55.23 M) NORTHWEST OF UTILITY POLE (17/426/9). |  |  |  |  |  |  |  |
| Notable Land marks: |  |  |  |  |  |  |  |

| SKETCH |  |
| --- | --- |